| Title | Document # | Location | Rev # | Effective Date |
| --- | --- | --- | --- | --- |
| Cleaning the Slushie Machine | STO-001 | Westmoreland & Jacktown | 0 | 02/05/2026 |

Cleaning the Slushie Machine at Westmoreland and Jacktown

1. Purpose

The purpose of this procedure is to establish requirements for daily

sanitizing and deep cleaning of the slushie machine to ensure food

safety, product quality, and equipment maintenance.

2. Scope

This procedure is applicable to all tasting room staff responsible for

slushie machine operation and maintenance with the machines at

Westmoreland or Jacktown.

3. Responsibilities

The responsibilities for the completion of this procedure are defined in

Section 5.

4. Definitions

Sanitizing: Quick sanitization procedure performed weekly or between

uses.

5. Procedure

Cleaning and Sanitizing** **with Nozzle Disassembly

Procedure /      Process Step / Activity        Responsible

Form                                                Function

------------------ ---------------------------------- ------------------

Empty machine of slushie contents  Tasting Room Staff

↓

Refill bowl with hot water and     Tasting Room Staff

wipe out. Empty and refill/rinse a

few times.

↓

Empty machine and keep pitcher     Tasting Room Staff

under nozzle to catch any leakage

while disassembling

↓

Put in a sanitizing packet with    Tasting Room Staff

fresh hot water

↓

Let sit for 30 minutes             Tasting Room Staff

↓

Drain the sanitizer solution       Tasting Room Staff

↓

Unscrew nut and pull pin out.      Tasting Room Staff

Slide tube and spring out. You

will have 4 parts: handle, top

cap, tube, and spring.

↓

Soak and clean all parts (handle,  Tasting Room Staff

top cap, tube, spring)

↓

Clean inside spout on the bowl     Tasting Room Staff

↓

Put spring back in tube, insert    Tasting Room Staff

into spout. Put top cap and handle

on. Handle hooks into spring

through tube opening. Slide pin

through and secure with nut. (May

require maneuvering to line up)